Supplement 1

| SOP Title: Genomic DNA Extraction from Fecal sample using the PowerSoil® DNA Isolation Kit |  |
| --- | --- |
| Authorized Signature: |  |
| Staff Able to Perform Procedure: Research Technician and higher |  |
| Manual: DNeasy PowerSoil Kit Handbook May 2017 |  |
| Principle of the Method: This protocol extracts genomic DNA from fecal samples at the UB Genomics and Bioinformatics Core. |  |
| Sample Type: Fecal Matter |  |
| Equipment Requirements: - Vortex (VWR) - Qiagen Vortex Adapter Tube holder (catalog #13000-V1-24) - Water bath for 2 ml tubes - Microcentrifuge (Eppendorf #5415D) - Pipette Man 2ul, 20ul, 200ul, 1000ul (Rainin LTS) - Filter Pipette Tips (Rainin LTS) - Timer |  |
| Reagents & Material Requirements: - PowerSoil® DNA Isolation Kit (Qiagen, #12888) - Nuclease Free Water (Fisher Scientific, catalog # AM9937) - Lo-Bind 1.5 ml tubes (Ambion, catalog # AM12450) - Cavicide - NucleoClean - 70% Ethanol |  |
| General Guidelines: - Fecal sample is frozen in matrix tubes. - Perform all centrifugation steps at room temperature (15-25°C). - Prepare samples in the Biosafety cabinet to prevent increased background. Incubations in water bath and centrifugation can be done outside the hood. - Wipe down all pipets and working surfaces before and after procedure with Cavicide and 70% EtOH and NucleoClean (pipets only) to prevent cross-contamination. - Check Solution C1. If Solution C1 is precipitated, heat solution at 60°C until dissolved before use. - Solution C6 contains no EDTA. To concentrate the DNA, see the Hints & Troubleshooting Guide in the PowerSoil manual. |  |
| Procedure - Thaw the fecal sample at RT (or 4oC overnight) and spin down (8,000 rpm, 3 min.) to settle material at bottom of tube. Before centrifugation, wrap the cover and tube with parafilm to prevent leaks. - Preweigh PowerBead tubes and tare the scale. To the PowerBead Tube, add approximately 50 mg (up to 250 mg) of fecal sample by using a sterile spatula (swab). Use the wooden end of swab to “scoop” enough material into PowerBead tube and rinse the end of the swab by swirling in the PowerBead tube. Record weight of each tube. - Add 60 μL of Solution C1 and vortex briefly. ***note: C1 solution can be added to the PowerBead tube prior to addition of fecal sample*** - Incubate at 65°C for 10 minutes. - Place the PowerBead Tubes in the Vortex Adapter Tube holder attached to vortexer. Ensure that the tubes are held tight. Vortex for 10 minutes at maximum speed. - Centrifuge tubes at 10,000 x g for 1 minute at room temperature. Make sure the PowerBead Tubes rotate freely in your centrifuge without rubbing. CAUTION: Be sure not to exceed 10,000 x g or tubes may break. - Add 250 μL of Solution C2 to a clean 2 mL Collection Tube (provided). Transfer up to 500 μL the supernatant to the 2 mL Collection Tube. Expect between 400-500 μL of supernatant at this step. - Vortex 5 seconds. Incubate at 4°C (on ice) for 5 minutes. - Centrifuge the tubes at room temperature for 1 minute at 10,000 x g. - Add 200 μL of Solution C3 to a clean 2 mL Collection Tube (provided). Avoiding the pellet, transfer up to, but no more than, 600 μL of supernatant into the 2 mL Collection Tube. - Vortex briefly. Incubate at 4°C for 5 minutes. - Centrifuge the tubes at room temperature for 1 minute at 10,000 x g. - Avoiding the pellet, transfer up to, but no more than, 600 μL of supernatant into a clean 2 mL Collection Tube (provided). - Shake to mix Solution C4 before use. Add 1200 μL of Solution C4 to a clean 2 ml tube (provided) then the supernatant from step 13. Vortex for 5 seconds. - Load approximately 680 μL of supernatant and C4 solution onto a Spin Filter and centrifuge at 10,000 x g for 1 minute at room temperature. Discard the flow through and add an additional 680 μL of supernatant to the Spin Filter and centrifuge at 10,000 x g for 1 minute at room temperature. Load the remaining supernatant onto the Spin Filter and centrifuge at 10,000 x g for 1 minute at room temperature. - Add 500 μL of Solution C5 and centrifuge at room temperature for 30 seconds at 10,000 x g. - Discard the flow through. - Centrifuge again at room temperature for 1 minute at 10,000 x g. - Carefully place spin filter in a clean 2 ml Collection Tube (provided). Avoid transferring any C5 Solution onto the Spin Filter. - Add 100 μL of Solution C6 to the center of the white filter membrane. - Centrifuge at room temperature for 30 seconds at 10,000 x g. - Discard the Spin Filter. The DNA in the tube is now ready for any downstream application. No further steps are required. Store DNA at -20oC or -80oC since C6 solution does not contain EDTA. |  |
| Quality Control of Samples - Check the concentration of the samples using the Invitrogen High Sensitivity DNA Qubit Quality Control using Qubit Fluorometer 3.0 SOP (for low numbers of samples) or the Invitrogen High Sensitivity DNA Quant IT Quality Control using Microplate Reader/Imager (for 20 or more samples). Log concentrations into workbooks stored on RT system. |  |

Supplement 2

| SOP Title: Illumina 16S V1-V3 Metagenomic Sequencing Library preparation (High Microbial Load) | Version 1.1, Page 1 |
| --- | --- |
| Authorized Signature: | Issuing Date: 09.11.14 |
|  | Last Revision Date: 09-11-2014 - MT |
| Staff Able to Perform Procedure: Research Technician and higher |  |
| Principle of the Method: This protocol explains how to convert metagenomic DNA samples into a library of template molecules suitable for subsequent cluster generation and sequencing using the Illumina 16S V1-V3 protocol and MiSeq (2x300) sequencer. |  |
| Sample Type: The Illumina 16S protocol has been optimized and validated using the V1-V3 primers. |  |
| Equipment Requirements: - Thermal cycler (Applied Biosystems) - Vortex (VWR) - Microcentrifuge (VWR) - Magnetic Stand (Ambion) - Timer (1 hour) |  |
| Reagents & Material Requirements: - 16S V1-V3 primers - 2x Kapa HiFi HotStart Ready Mix (Kapa Biosystems, part #KK2601) - Agencourt AMPure XP 60 ml kit (Beckman Coulter, Catalog # A63881) - Agarose - Ethidium Bromide - 100% Ethanol (VWR) - Ultra pure water (VWR) - Plate sealers or strip lids (VWR) - 96-well 0.4ml PCR plates (Applied Biosystems) - RNAse- free centrifuge tubes 1.5ml, 200 ul (USA Scientific) - Pipette Man 2ul, 20ul, 200ul, 1000ul (Rainin) - Filter Pipette Tips (Rainin) |  |
| STEP A: PCR I Amplification (Amplicon PCR) - For each sample, combine the following reagents on ice in the PCR plate. - 10.5 µl High-quality Metagenomic DNA (12.5 ng/reaction) - 1 µl 16S V1 Forward Primer 5 μM - 1 µl 16S V3 Reverse Primer 5 μM - 12.5 µl 2x Kapa HiFi HotStart Ready Mix 25 µl TOTAL - Set pipette to 25 µl, gently pipette the entire volume up and down 10 times. - Apply sealing film, spin down plate and place in thermocycler for the following PCR cycles. |  |
| - 3 min 95°C - 30 sec 95°C - 30 sec 62.3°C Repeat for 25 cycles - 30 sec 72°C - 5 min 72°C - ∞ 4°C |  |
| STEP B: PCR I Cleanup - Vortex the AMPure XB Beads for 30 sec. Add 20 µl of AMPure XP Beads to each clear sample and gently pipette the entire volume up and down 10 times. - Incubate at RT for 5 min. - Place the PCR tubes on the magnetic stand at RT until the supernatant appears completely clear (~ 5 min). - Gently remove and discard 43 μl supernatant taking care not to disturb the beads. Some liquid may remain in wells. - With tubes on stand, gently add 200 µl of freshly prepared 80% ethanol to each magnetic bead pellet and incubate plate at RT for 30 sec. Carefully remove ethanol by pipette. - Repeat step above, for a total of 2 ethanol washes. Ensure all ethanol has been removed. - With tubes on the magnetic stand, allow beads to air-dry at RT for 20 min. - Remove the tubes from the magnetic stand, and resuspend dried beads with 35 µl Resuspension Buffer. Gently, pipetted entire volume up and down 10 times mixing thoroughly. Ensure beads are no longer attached to the side of the well. - Incubate resuspended beads at RT for 2 min. - Place tubes on magnetic stand for 5 min until the sample appears clear. - Gently transfer 30 µl of clear supernatant (purified PCR I product) to new well. |  |
| STEP C: PCR II Amplification - For each sample, combine the following reagents. Use the Illumina Experimental Manager to generate correct index combinations. - 5 µl Purified PCR I product - 10 µl PCR Grade water (DNA-free) - 5 µl Nextera XT Index Primer 1 - 5 µl Nextera XT Index Primer 2 - 25 µl 2x KAPA HiFi HotStart Ready Mix 50 µl TOTAL - Set pipette to 50 µl, gently pipette the entire volume up and down 10 times. - Apply strip lids, spin down strip tubes and place in thermocycler for the following PCR cycles. - 3min 95°C - 30 sec 95°C - 30 sec 55°C Repeat for 8 cycles - 30 sec 72°C - 5 min 72°C - ∞ 4°C |  |
| STEP D: PCR II Cleanup - Vortex the AMPure XP Beads and add 56 µl of AMPure XP Beads to each clear sample. Gently pipette the entire volume up and down 10 times. - Incubate at RT for 5 min. - Place the PCR tubes on the magnetic stand at RT until the supernatant appears completely clear (~ 5 min). - Gently remove and discard the supernatant taking care not to disturb the beads. Some liquid may remain in wells. - With tubes on stand, gently add 200 µl of freshly prepared 80% ethanol to each magnetic bead pellet and incubate plate at RT for 30 sec. Carefully remove ethanol by pipette. - Repeat step above, for a total of 2 ethanol washes. Ensure all ethanol has been removed. - With tubes on the magnetic stand, allow beads to air-dry at RT for 20 min. - Remove the tubes from the magnetic stand and resuspend dried beads with 22.5 µl Resuspension Buffer. Gently, pipetted entire volume up and down 10 times mixing thoroughly. Ensure beads are no longer attached to the side of the well. - Incubate resuspended beads at RT for 2 min. - Place tubes on magnetic stand for 5 min until the sample appears clear. - Gently transfer 20 µl of clear supernatant (purified PCR II product) to new well. |  |
| Validate Library - Quantify the samples using Picogreen Assay (SOP “qc_ Picogreen_Assay”). Library QC should ideally be above 60 ng/ul. - Quantify by running smaples on a 2% gel; load 2 µl of 6X Gel loading Dye, 6 ul of PCR II Product and 5 µl 100 bp plus DNA ladder. Alternatively, use the Bioanalyzer High Sensitivity chip. You expect to see a ~ 650 bp band. Converting ng\ul to nM |  |